- Additional Files to be Added

- In properties of the file

Include Options:

Predefined Symbols:

File Search Path:

Gineral:

Add All these and cross check if all these options are opted in the properties

Output:

Under View check for terminal option
there give
COM(N)  based on ur pc connections

Baud rate 115200

Data size:8

Parity None
stop bits: 1

And click ok to run those in terminal to view it 

Instead u can run in putty and save directly in CSV and sample the the adc values

To RUN

FIRST build ur projects and then push it into the board

ADCPIN: Pin no 23

Ground: GND

Changed code with better time stamp and ADC Sample output

//###########################################################################

//

// FILE:    Example_F28379D_LaunchPadDemo.c

//

// TITLE:   F28379D LaunchPad Out of Box Demo

//

// DESCRIPTION:

//! \addtogroup cpu01_example_list

//! <h1>Out of Box Demo (LaunchPadDemo)</h1>

//!

//!  This program is the demo program that comes pre-loaded on the F28379D

//!  LaunchPad development kit.  The program starts by flashing the two user

//!  LEDs. After a few seconds the LEDs stop flashing and the device starts

//!  sampling ADCIN14 once a second.  If the sample is greater than midscale

//!  the red LED on the board is lit, while if it is lower a blue LED is lit.

//!  Sample data is also display in a serial terminal via the boards back

//!  channel UART.  You may view this data by configuring a serial terminal

//!  to the correct COM port at 115200 Baud 8-N-1.

//!

//

//###########################################################################

//

//

// $Copyright:

// Copyright (C) 2013-2025 Texas Instruments Incorporated - http://www.ti.com/

//

// Redistribution and use in source and binary forms, with or without

// modification, are permitted provided that the following conditions

// are met:

//

//   Redistributions of source code must retain the above copyright

//   notice, this list of conditions and the following disclaimer.

//

//   Redistributions in binary form must reproduce the above copyright

//   notice, this list of conditions and the following disclaimer in the

//   documentation and/or other materials provided with the

//   distribution.

//

//   Neither the name of Texas Instruments Incorporated nor the names of

//   its contributors may be used to endorse or promote products derived

//   from this software without specific prior written permission.

//

// THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND CONTRIBUTORS

// "AS IS" AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT

// LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR

// A PARTICULAR PURPOSE ARE DISCLAIMED. IN NO EVENT SHALL THE COPYRIGHT

// OWNER OR CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL,

// SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT

// LIMITED TO, PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE,

// DATA, OR PROFITS; OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY

// THEORY OF LIABILITY, WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT

// (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE USE

// OF THIS SOFTWARE, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

// $

//###########################################################################

//

// Included Files

//

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

#include <file.h>

#include "F28x_Project.h"     // DSP28x Headerfile

#include "ti_ascii.h"

#include "sci_io.h"

//

// Defines

//

//

// Micro-seconds to wait for ADC conversion. Longer than necessary.

//

#define CONV_WAIT 1L

//

// Globals

//

extern void DSP28x_usDelay(Uint32 Count);

static unsigned short indexX=0;

static unsigned short indexY=0;

const unsigned char escRed[] = {0x1B, 0x5B, '3','1', 'm'};

const unsigned char escWhite[] = {0x1B, 0x5B, '3','7', 'm'};

const unsigned char escLeft[] = {0x1B, 0x5B, '3','7', 'm'};

const unsigned char pucTempString[] = "ADCIN14 Sample:     ";

int16_t currentSample;

//

// sampleADC -

//

int16_t sampleADC(void)

{

int16_t sample;

//

// Force start of conversion on SOC0

//

AdcaRegs.ADCSOCFRC1.all = 0x03;

//

// Wait for end of conversion.

//

while(AdcaRegs.ADCINTFLG.bit.ADCINT1 == 0)

{

//

// Wait for ADCINT1

//

}

AdcaRegs.ADCINTFLGCLR.bit.ADCINT1 = 1;        // Clear ADCINT1

//

// Get ADC sample result from SOC0

//

sample = AdcaResultRegs.ADCRESULT1;

return(sample);

}

//

// drawTILogo -

//

void drawTILogo(void)

{

unsigned char ucChar, lastChar;

putchar('\n');

while(indexY<45)

{

if(indexY<45)

{

if(indexX<77)

{

ucChar = ti_ascii[indexY][indexX++] ;

//

// We are in the TI logo make it red

//

if(ucChar != '7' && lastChar=='7')

{

putchar(escRed[0]);

putchar(escRed[1]);

putchar(escRed[2]);

putchar(escRed[3]);

putchar(escRed[4]);

}

//

// We are in the TI logo make it red

//

if(ucChar == '7' && lastChar!='7')

{

putchar(escWhite[0]);

putchar(escWhite[1]);

putchar(escWhite[2]);

putchar(escWhite[3]);

putchar(escWhite[4]);

}

putchar(ucChar);

lastChar = ucChar;

}

else

{

ucChar = 10;

putchar(ucChar);

ucChar = 13;

putchar(ucChar);

indexX=0;

indexY++;

}

}

}

}

//

// clearTextBox -

//

void clearTextBox(void)

{

putchar(0x08);

//

// Move back 24 columns

//

putchar(0x1B);

putchar('[');

putchar('2');

putchar('6');

putchar('D');

//

// Move up 3 lines

//

putchar(0x1B);

putchar('[');

putchar('3');

putchar('A');

//

// Change to Red text

//

putchar(escRed[0]);

putchar(escRed[1]);

putchar(escRed[2]);

putchar(escRed[3]);

putchar(escRed[4]);

printf((char*)pucTempString);

//

// Move down 1 lines

//

putchar(0x1B);

putchar('[');

putchar('1');

putchar('B');

//

// Move back 20 columns

//

putchar(0x1B);

putchar('[');

putchar('2');

putchar('0');

putchar('D');

//

// Save cursor position

//

putchar(0x1B);

putchar('[');

putchar('s');

}

//

// updateDisplay -

//

void updateDisplay(void)

{

//

// Restore cursor position

//

putchar(0x1B);

putchar('[');

putchar('u');

printf("%d               ", currentSample);

}

//

// scia_init - SCIA  8-bit word, baud rate 0x000F, default, 1 STOP bit,

// no parity

//

void scia_init()

{

//

// Note: Clocks were turned on to the SCIA peripheral

// in the InitSysCtrl() function

//

//

// 1 stop bit,  No loopback, No parity,8 char bits, async mode,

// idle-line protocol

//

SciaRegs.SCICCR.all =0x0007;

//

// enable TX, RX, internal SCICLK, Disable RX ERR, SLEEP, TXWAKE

//

SciaRegs.SCICTL1.all =0x0003;

SciaRegs.SCICTL2.bit.TXINTENA =1;

SciaRegs.SCICTL2.bit.RXBKINTENA =1;

//

// 115200 baud @LSPCLK = 50MHz (200MHz SYSCLK)

//

SciaRegs.SCIHBAUD.all    =0x0000;

SciaRegs.SCILBAUD.all    =53;

SciaRegs.SCICTL1.all =0x0023;  // Relinquish SCI from Reset

return;

}

//

// Main

//

void main()

{

volatile int status = 0;

uint16_t i;

volatile FILE *fid;

//

// If running from flash copy RAM only functions to RAM

//

#ifdef _FLASH

memcpy(&RamfuncsRunStart, &RamfuncsLoadStart, (size_t)&RamfuncsLoadSize);

#endif

//

// Initialize System Control:

// PLL, WatchDog, enable Peripheral Clocks

// This example function is found in the F2806x_SysCtrl.c file.

//

InitSysCtrl();

//

// For this example, only init the pins for the SCI-A port.

//

EALLOW;

GpioCtrlRegs.GPBMUX1.bit.GPIO42 = 3;

GpioCtrlRegs.GPBMUX1.bit.GPIO43 = 3;

GpioCtrlRegs.GPBGMUX1.bit.GPIO42 = 3;

GpioCtrlRegs.GPBGMUX1.bit.GPIO43 = 3;

EDIS;

//

// Clear all interrupts and initialize PIE vector table:

// Disable CPU interrupts

//

DINT;

//

// Initialize PIE control registers to their default state.

// The default state is all PIE interrupts disabled and flags

// are cleared.

// This function is found in the F2837xD_PieCtrl.c file.

//

InitPieCtrl();

//

// Disable CPU interrupts and clear all CPU interrupt flags

/*

IER = 0x0000;

IFR = 0x0000;

*/

//

// Initialize the PIE vector table with pointers to the shell Interrupt

// Service Routines (ISR).

// This will populate the entire table, even if the interrupt

// is not used in this example.  This is useful for debug purposes.

// The shell ISR routines are found in F2837xD_DefaultIsr.c.

// This function is found in F2837xD_PieVect.c.

//

InitPieVectTable();

//

// Initialize SCIA

//

scia_init();

//

// Initialize GPIOs for the LEDs and turn them off

//

EALLOW;

GpioCtrlRegs.GPADIR.bit.GPIO31 = 1;

GpioCtrlRegs.GPBDIR.bit.GPIO34 = 1;

GpioDataRegs.GPADAT.bit.GPIO31 = 1;

GpioDataRegs.GPBDAT.bit.GPIO34 = 1;

EDIS;

//

// Enable global Interrupts and higher priority real-time debug events:

//

EINT;   // Enable Global interrupt INTM

ERTM;   // Enable Global realtime interrupt DBGM

//

// Configure the ADC: Initialize the ADC

//

EALLOW;

//

// write configurations

//

AdcaRegs.ADCCTL2.bit.PRESCALE = 6;      // set ADCCLK divider to /4

AdcbRegs.ADCCTL2.bit.PRESCALE = 6;      // set ADCCLK divider to /4

AdcSetMode(ADC_ADCA, ADC_RESOLUTION_12BIT, ADC_SIGNALMODE_SINGLE);

AdcSetMode(ADC_ADCB, ADC_RESOLUTION_12BIT, ADC_SIGNALMODE_SINGLE);

//

// Set pulse positions to late

//

AdcaRegs.ADCCTL1.bit.INTPULSEPOS = 1;

AdcbRegs.ADCCTL1.bit.INTPULSEPOS = 1;

//

// power up the ADCs

//

AdcaRegs.ADCCTL1.bit.ADCPWDNZ = 1;

AdcbRegs.ADCCTL1.bit.ADCPWDNZ = 1;

//

// delay for 1ms to allow ADC time to power up

//

DELAY_US(1000);

//

// ADCA

//

EALLOW;

AdcaRegs.ADCSOC0CTL.bit.CHSEL = 0x0E;   //SOC0 will convert pin ADCIN14

//

// sample window is acqps + 1 SYSCLK cycles

//

AdcaRegs.ADCSOC0CTL.bit.ACQPS = 25;

AdcaRegs.ADCSOC1CTL.bit.CHSEL = 0x0E;   //SOC1 will convert pin ADCIN14

//

// sample window is acqps + 1 SYSCLK cycles

//

AdcaRegs.ADCSOC1CTL.bit.ACQPS = 25;

AdcaRegs.ADCINTSEL1N2.bit.INT1SEL = 1;  //end of SOC1 will set INT1 flag

AdcaRegs.ADCINTSEL1N2.bit.INT1E = 1;    //enable INT1 flag

AdcaRegs.ADCINTFLGCLR.bit.ADCINT1 = 1;  //make sure INT1 flag is cleared

//

// Redirect STDOUT to SCI

//

status = add_device("scia", _SSA, SCI_open, SCI_close, SCI_read, SCI_write,

SCI_lseek, SCI_unlink, SCI_rename);

fid = fopen("scia","w");

freopen("scia:", "w", stdout);

setvbuf(stdout, NULL, _IONBF, 0);

//

// Print a TI Logo to STDOUT

//

drawTILogo();

//

// Twiddle LEDs

//

GpioDataRegs.GPADAT.bit.GPIO31 = 0;

GpioDataRegs.GPBDAT.bit.GPIO34 = 1;

for(i = 0; i < 50; i++)

{

GpioDataRegs.GPATOGGLE.bit.GPIO31 = 1;

GpioDataRegs.GPBTOGGLE.bit.GPIO34 = 1;

DELAY_US(50000);

}

//

// LEDs off

//

GpioDataRegs.GPADAT.bit.GPIO31 = 1;

GpioDataRegs.GPBDAT.bit.GPIO34 = 1;

//

// Clear out one of the text boxes so we can write more info to it

//

clearTextBox();

currentSample = sampleADC();

//

// Main program loop - continually sample temperature

//

for(;;)

{

currentSample = sampleADC();

static unsigned long time_us = 0;

const unsigned long Ts_us = 1;

printf("%lu, %d\n", time_us, currentSample);

time_us += Ts_us;

// LED indication (optional)

if(currentSample > 2048)

{

GpioDataRegs.GPBSET.bit.GPIO34 = 1;

GpioDataRegs.GPACLEAR.bit.GPIO31 = 1;

}

else

{

GpioDataRegs.GPBCLEAR.bit.GPIO34 = 1;

GpioDataRegs.GPASET.bit.GPIO31 = 1;

}

DELAY_US(1);  // adjust this if your sample period is different

}

}

//

// End of File

//